CSC258 LAB2 PRELAB

TINGFENG XIA, 1003780884, xiatingf

5 PART I

- This is a screen shot of the output.

Our inputs are SW[0], SW[1], SW[9] and output is sent to LEDR[0], we ran each combination of input for 10ns. We have the output as follows:

| SW[0] (x) | SW[1] (y) | SW[9] (s) | LEDR[0] (m) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 |

According to the results above, we can confirm that the design is working properly since when SW[9], i.e. input s, is zero, the output m is equal to SW[0] which is our x input and when SW[9] is 1, the output m is equal to the y input(SE[1]). So the multiplexer is correct.

6 PART II

- Here we have 6 inputs in total to consider, so there would be 26=64 rows in a full truth table.

- Here is my design (Notice that I assumed the mux2to1 multiplexer to have the same input/output layout as specified in the handout.)

- Please see the mux4.v file.

- Please see the mux4.do file for the ModelSim code, here is the output

The result is consistent with my design. Notice that by the way I designed my do file, the first 40ns output wave should look like the first input(u, SW[0]), the second 40ns output should look like the second input(V, SW[1]), the third 40ns input should look like the third input(w, SW[2]) and the last 40ns input should look like the fourth input(x, SW[3]). The output that I got from LEDR[0] is exactly what I expected so I am confident that my circuit is working correctly.

7 PART III

- First, let’s write down the full truth table (Notice that here 0 marks emitting light)

| hex. | C3 | C2 | C1 | C0 | HEX[0] | HEX[1] | HEX[2] | HEX[3] | HEX[4] | HEX[5] | HEX[6] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 4 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 6 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| A | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| B | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| C | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| D | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| E | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| F | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |

#NOTATION: For the following 7 tables and expressions, by AB  I mean AB, this was to simplify the typesetting.

- For HEX[0], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 0 | 1 | 0 | 0 |
| c3c2 | 1 | 0 | 0 | 0 |
| c3c2 | 0 | 1 | 0 | 0 |
| c3c2 | 0 | 0 | 1 | 0 |

Thus, HEX0=c3c2c1c0+c3c2c1c0+c3c2c1c0+c3c2c1c0

- For HEX[1], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 0 | 0 | 0 | 0 |
| c3c2 | 0 | 1 | 0 | 1 |
| c3c2 | 1 | 0 | 1 | 1 |
| c3c2 | 0 | 0 | 1 | 0 |

Thus, HEX1=c3c2c1c0+c3c1c0+c2c1c0+c3c2c0

- For HEX[2], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 0 | 0 | 0 | 1 |
| c3c2 | 0 | 0 | 0 | 0 |
| c3c2 | 1 | 0 | 1 | 1 |
| c3c2 | 0 | 0 | 0 | 0 |

Thus, HEX2= c3c2c0+c3c2c1+  c3c2c1co

- For HEX[3], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 0 | 1 | 0 | 0 |
| c3c2 | 1 | 0 | 1 | 0 |
| c3c2 | 0 | 0 | 1 | 0 |
| c3c2 | 0 | 0 | 0 | 1 |

Thus, HEX3=c3c2c1c0+ c3c2c1c0+c2c1c0+c3c2c1c0

- For HEX[4], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 0 | 1 | 1 | 0 |
| c3c2 | 1 | 1 | 1 | 0 |
| c3c2 | 0 | 0 | 0 | 0 |
| c3c2 | 0 | 1 | 0 | 0 |

Thus, HEX4=c3c0+c2c1c0+c3c2c1

- For HEX[5], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 0 | 1 | 1 | 1 |
| c3c2 | 0 | 0 | 1 | 0 |
| c3c2 | 0 | 1 | 0 | 0 |
| c3c2 | 0 | 0 | 0 | 0 |

Thus, HEX5=c3c2c0+c3c2c1+c3c1c0+c3c2c1c0

- For HEX[6], we have the Karnaugh map

|  | c1c0 | c1c0 | c1c0 | c1co |
| --- | --- | --- | --- | --- |
| c3c2 | 1 | 1 | 0 | 0 |
| c3c2 | 0 | 0 | 1 | 0 |
| c3c2 | 1 | 0 | 0 | 0 |
| c3c2 | 0 | 0 | 0 | 0 |

Thus, HEX6= c3c2c1+c3c2c1c0+c3c2c1c0

- Please see the hexdecoder.v file